SECTION 10 73 16

ALUMINUM CANOPIES

Marquee Canopies & Awnings by CityScapes

Copyright 2025 CityScapes® International, Inc. - All rights reserved

** NOTE TO SPECIFIER **  CityScapes Inc.; Aluminum Canopies

This section is based on the products of CityScapes International Inc., which is located at:

4200 Lyman Ct.

Hilliard, OH 43026

Toll Free:  877-SCREENS

Phone:  614-850-2549

Email:  Contact@cityscapesinc.com.

Web:  https://cityscapesinc.com/

CityScapes® has been an industry leader in architectural innovations since 2004. We began with our flagship product, Envisor®, a patented HVAC Screening system that meets municipal code line-of-sight requirements with no roof membrane disruption. CityScapes® has expanded its scope to add new product lines that combine screening, security, and aesthetics.  Our offerings include:

•     Envisor® Unit Attached Rooftop Screens

•     Envisor® Z-Fin and post-mounted screening,

•     Covrit® Ground Screening and Roofed Enclosures

•     ToughGate™ and MegaGate™

•     Marquee Canopies and Awnings

•     NatureScreen Trellises® and Planters

•     Custom fabrication solutions

All CityScapes® products are Build America, Buy America Act (BABAA) certified and constructed at our 180,000 square foot manufacturing facility in central Ohio. Design, engineering, manufacturing, and installation - our team handles it all.

GENERAL

SECTION INCLUDES

Aluminum canopies attached to the building.

REFERENCE STANDARDS

ASTM International: (ASTM):  ASTM B 221 - Aluminum and Aluminum Alloy Extruded Bars, Rods, Wire Profiles, and Tubes.

American Society of Civil Engineers (ASCE):  ASCE 7 - Minimum Design Loads for Buildings and Other Structures.

SUBMITTALS

Submit under provisions of Division 01.

Product Data:

Manufacturer's data sheets on each product to be used.

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Typical installation methods.

Sufficient data and detail to indicate compliance with these specifications.

Shop Drawings:  Prior to commencement of fabrication, submit detailed shop drawings, showing profiles, sections of components, finishes, and fastening details. Include plan elevation pages to clearly outline canopy locations.

Color Charts:  Showing manufacturer’s full range of colors from standard line.

Certification:  Manufacturer's Certificate of Compliance certifying that panels supplied meet or exceed requirements specified.

Closeout Submittals:  Warranty documents, issued and executed by manufacturer, countersigned by Contractor.

QUALITY ASSURANCE

Manufacturer Qualifications:  Company specializing in manufacturing products specified in this section with a minimum two years documented experience.

Installer Qualifications:  Company specializing in performing Work of this section with minimum two years documented experience with projects of similar scope and complexity.

Source Limitations:  Provide each type of product from a single manufacturing source to ensure uniformity.

DELIVERY, STORAGE, AND HANDLING

Delivery:  Deliver materials to site in manufacturer's original, unopened containers and packaging, with labels clearly indicating manufacturer and material.

Deliver materials to project site ready for erection.

Storage and Handling:  Protect materials and finishes during handling and installation to prevent damage.

Protection:  Protect from damage due to weather, excessive temperature, and construction operations.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

Field Measurements:  Take measurements of supporting structure. Indicate measurements on shop drawings fully documenting any field condition that may interfere with the system installation.

COORDINATION

Installer for work under this Section shall be responsible for coordination of panel and framing sizes and required options with the Drawings and Contractor's requirements.

Confirm size, type, and location of supporting construction as adequate to resist loads imposed by canopy.

WARRANTY

Warranty:  Manufacturer's standard limited warranty in which manufacturer agrees to repair or replace components that fail in materials or workmanship within specified warranty period.

Warranty Period:  Three years from date of Substantial Completion.

PRODUCTS

MANUFACTURERS

Basis-of-Design:  Marquis Canopies by CityScapes International Inc; 4200 Lyman Ct., Hilliard, OH 43026; Toll Free:  877-SCREENS; Phone:  614-850-2549; Email:  contact@cityscapesinc.com; www.cityscapesinc.com.

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

Substitutions:  Not permitted.

Requests for substitutions will be considered in accordance with provisions of Division 01.

PERFORMANCE AND DESIGN REQUIREMENTS

Regulatory Requirements:  Comply with requirements of building authorities having jurisdiction in Project location.

Design Criteria:  Manufacturer is responsible for the structural design of all materials, assembly, and attachments to resist snow, wind, suction, and uplift loading at any point without damage or permanent set.

Framing:  Designed in accordance with the Aluminum Design Manual to resist the following loading:

ASCE 7 - Minimum Design Loads for Buildings and Other Structures; American Society of Civil Engineers.

Configuration:  Layout and dimensions, Fascia profile, and roof covering design as indicated on drawings.

Installation:  Face mounted to building structure with or without hanger rods as specified in drawings.

Drainage Concept:  Water collected in decking conducted into perimeter fascia gutter and discharged through scupper as specified in drawings.

MATERIALS

Framing:  Aluminum extrusions, sheet, or tube.

Hanger Rods:  Zinc plated steel and powder coat. Prime and paint are not acceptable.

Connections:  Aluminum wall plates and canopy mounting brackets.

Hardware and Fasteners:  Nuts, bolts, washers, clevis pins, screws, anchors, and pipe spacers to be zinc plated or galvanized steel required to suit application.

Flashing and Z-Closure:  Match skin material, fabricated to prevent leakage, and sealed with metal roof sealant in clear or color match.

Finish:  Powder coat finish complying with finish manufacturer’s written instructions for surface preparation including pretreatment, application, baking, and minimum dry film thickness. Color to be selected from manufacturer’s standard color line.

Covering:

Interlocking extruded aluminum decking modules.

Fascia:  Manufacturer's standard flat face profile.

Size:  8, 10 or 12 inches height as indicated on the Drawings.

Exposed Scupper:  Color to match canopy covering, manufacturer’s recommended size for canopy specified.

Integrated Light Fixtures:  Provide prepared openings and required access for field installed lighting and electrical components as per fixture manufacturer standard drawings and locations as specified in drawings,

SHOP FABRICATION

Provide a complete system ready for erection at project site.  Field verify dimensions of supporting structure at site of installation prior to fabrication.

Shop fabricate to the greatest extent possible; disassemble as necessary for shipping.  Clearly mark units for reassembly and coordinated installation.

Fabricate connections for bolt, nut, and washer connectors.

FINISHES

Aluminum Framing and Decking:  Manufacturer's standard powder coating system, factory applied.

ACCESSORIES

Trim, Closure Pieces, and Flashings:  Same material, thickness, and finish as sheet metal decking; factory-fabricated to required profiles.

Fasteners:  Stainless steel or zinc coated for corrosion resistance.

EXECUTION

EXAMINATION

Installer's Examination:  Examine conditions under which construction activities of this section are to be performed.

Submit written notification to Architect and manufacturer if such conditions are unacceptable.

Beginning erection constitutes installer's acceptance of conditions.

PREPARATION

Clean surfaces thoroughly prior to installation.

Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

INSTALLATION

Install canopies per manufacturer’s written instructions and as indicated on approved shop drawings.

Locate and place canopies level, plumb and at indicated alignment with adjacent work.

Anchor system to building components; provide adequate clearance for movement caused by thermal expansion and contraction and wind loads.

Do not field cut or alter structural members without approval.

Touch-up damaged finish coating using material provided by manufacturer to match original coating.

Protect galvanized and nonferrous-metal surfaces from corrosion or galvanic action by applying a coating of bituminous paint or elastomeric coating on surfaces that will be in contact with concrete, masonry, or dissimilar metals.

ERECTION TOLERANCES

Tolerances:  Maximum misalignment from true position 1/4 inch (6 mm).

CLEANING AND PROTECTION

Remove protective masking from material immediately after installation.

Protection:

Ensure that finishes and structure of installed systems are not damaged by subsequent construction activities.

If minor damage to finishes occurs, repair damage in accordance with manufacturer's recommendations; provide replacement components if repaired finishes are unacceptable to Architect.

Prior to Substantial Completion:  Remove dust or other foreign matter from component surfaces; clean finishes in accordance with manufacturer's instructions.

Clean units in accordance with the manufacturer's instructions.

END OF SECTION